ENVIRONMENTAL CONTROL

# Purpose

To ensure that the environmental conditions within the laboratory facility are monitored, controlled, and maintained to prevent contamination, degradation, or variation in reference material properties.

# Scope

Applies to all production, testing, packaging, and storage areas where reference materials (RMs or CRMs), client’s coal samples are handled.

# References

- ISO 17034:2016, Clauses 6.4, 7.4, 7.5, 7.6, 7.7, 7.8

- ISO/IEC 17025:2017, Clauses 6.3 and 6.4

- Internal SOPs on Material Handling, Stability Studies, and Quality Control

# Responsibilities

| Role | Responsibility |
| --- | --- |
| Laboratory Manager | Implement SOPs for Environmental conditions |
| Laboratory Supervisor | Ensure compliance with environmental requirements |
| Laboratory Analyst | Perform routine checks, record conditions |
| Maintenance Personnel | Maintain heating, Ventilation and Air-conditioning (HVAC). |

# Control Environmental Parameters

| Parameter | Acceptable Range | Controlled Areas |
| --- | --- | --- |
| Temperature | 19-26 oC | All preparation, weighing, and packaging |
| Humidity | 25-60 % RH (unless specified), for the analysis room. Storage 16-20 oC | Drying, crushing, packaging, analysis. |
| Particulate | Minimal visible particles | Cleanroom or clean bench |
| Airflow | Positive pressure if required | Clean areas |
| Light | Avoid direct sunlight | Storage and packaging |

# Environmental Control Procedure

  - Daily Monitoring

- Check and record temperature and humidity before starting with work and closely monitor these throughout the shift.

- Use calibrated hygrometers and thermometers

  - Cleaning and Sanitation

- Clean benches, scales, and packaging areas daily using disinfectants.

- Floors and walls cleaned as needed.

  - Pest and Contamination Control

- Materials must be kept in closed, labelled containers.

- Food and drinks strictly prohibited in lab and storage areas.

# Control of Non-conforming Conditions

- Inform supervisor and stop critical activities

- Label the affected area/equipment as “Do not use”.

- Conduct the impact assessment for affected area.

- Complete a nonconformance report and initiate corrective action.

# Record Keeping

| Record Name | Retention period (in years) | Stored by |
| --- | --- | --- |
| Daily environmental log | 5 | Laboratory Manager |
| Data Logger reports | 5 | IT/LIMS |
| Non-conformance report | 10 | Operations Manager |

# Training

All staff working in controlled areas must be trained on:

- Good environmental hygiene

- SOP for environmental monitoring

Training must be documented and environmental conditions included in SOPs.

# Review and updates

SOP to be reviewed annually or when:

- A new environmental control system is implemented

- If there is a deviation that impacts the quality of product

- Regulatory or ISO requirements become updated.