# Standard Operating Procedure (SOP)

# Title: Receiving and Inspecting Raw Materials (Coal Samples, Equipment, Reagents, and Chemicals)

Document Number: 7.4.1
Effective Date: 22/07/2025
Review Date: 22/07/2026
Prepared by: Sifiso Nyambi
Approved by: Nonhlanhla Msibi

## 1. Purpose

To define the procedure for receiving, inspecting, and documenting all raw materials required by the coal laboratory, including laboratory-collected coal samples, purchased equipment, reagents, and chemicals. This ensures that materials are fit for use, traceable, and handled safely in compliance with quality standards.

## 2. Scope

This SOP applies to:
- Coal samples collected by the laboratory from mines, stockpiles, or client sites.
- Equipment, reagents, consumables, and chemicals purchased from suppliers.
- Any externally supplied coal or reference standards.

## 3. Responsibilities

- Sample Collection Team / Field Staff: Ensure integrity, labeling, and documentation of coal samples before delivery to the lab.
- Lab Technician / Receiver: Inspect delivered samples and purchased items, record details, and segregate accepted vs. non-conforming materials.
- Laboratory Supervisor: Review inspection records and approve materials for use.
- Laboratory Manager: Authorize final acceptance, resolve discrepancies, and liaise with suppliers/clients.

## 4. Definitions

- Coal Sample: Representative material collected for laboratory analysis.
- Raw Materials: Includes coal samples, reagents, consumables, chemicals, and laboratory equipment.
- COA (Certificate of Analysis): Document from suppliers stating quality specifications for purchased reagents/chemicals.

## 5. Materials and Equipment Needed

- Receiving logbook / electronic inventory system
- PPE (gloves, lab coat, dust mask, goggles)
- Inspection checklist forms (coal sample form, purchased material form)
- Labels/barcodes for traceability
- Quarantine area for non-conforming items

## 6. Procedure

### 6.1 Receiving Coal Samples (Laboratory-Collected)

1. Confirm sample ID, source location, date/time of collection, and collector’s details.
2. Inspect containers/bags for:
   - Proper sealing to prevent contamination or spillage.
   - Integrity (no breakage, moisture infiltration, or mislabeling).
3. Record details in the Coal Sample Receiving Log.
4. Label each sample with an internal laboratory code.
5. Transfer accepted samples to the designated storage/preparation area.
6. If discrepancies (damaged, unlabeled, contaminated samples) are found, place them in quarantine and notify the Laboratory Manager.

### 6.2 Receiving Purchased Materials (Equipment, Reagents, Chemicals, Consumables)

1. Match the delivery note and invoice against the purchase order (PO).
2. Verify supplier documents (including COA for chemicals/reagents, calibration certificates for equipment, if applicable).
3. Inspect items for:
   - Packaging integrity (no leakage, corrosion, breakage).
   - Correct labeling (name, batch/lot number, expiry date, hazard warnings).
   - Compliance with specifications (grade, concentration, size, capacity).
4. Record details in the Purchased Materials Receiving Log.
5. Label accepted items with internal inventory code and transfer to designated storage.
6. If defective or non-conforming, segregate and report to the Laboratory Supervisor and Laboratory Manager for resolution with the supplier.

### 6.3 Documentation

1. Complete inspection forms for each received batch of samples/materials.
2. Maintain records of all deliveries in logbooks or digital systems.
3. Attach supporting documents (COA, calibration certificates, delivery notes).
4. Update laboratory inventory/register immediately after acceptance.

## 7. Safety and Precautions

- Always wear appropriate PPE when handling coal samples or chemicals.
- Prevent cross-contamination by handling coal samples in a clean, designated area.
- Follow chemical safety SOPs for hazardous substances.
- Handle heavy equipment deliveries using correct lifting techniques or mechanical aids.

## 8. Records

- Coal Sample Receiving Log
- Purchased Materials Receiving Log
- Inspection Checklists (coal / purchased)
- Certificates of Analysis (for chemicals/reagents)
- Non-Conformance Reports

## 9. References

- ISO/IEC 17025:2017, ISO 17034, ISO 17043 – General requirements for the competence of testing and calibration laboratories
- Laboratory Quality Manual
- Supplier Specifications and Client Sample Submission Guidelines

## 10. Revision History

Revision; Date; Description of Change; Approved By. 0; 22/07/2025; Initial issue; Nonhlanhla Msibi.