Applies to: All stages of Proficiency Testing and reference material (RM or CRM) production including sampling, characterization, homogeneity, stability, certification, labelling, and distribution.

## 1. Purpose

To define the procedure for identifying, documenting, evaluating, correcting, and preventing recurrence of any non-conforming work (NCW) that could affect the quality, integrity, or validity of certified reference materials (CRMs) or candidate RMs.

## 2. Scope

This applies to:

- RM/CRM batches failing stability or homogeneity requirements

- Incorrect property value assignments or uncertainty estimations

- Equipment malfunction affecting measurement validity

- Labelling, packaging, or CoA errors

- Any deviation from production or testing procedures

- Incorrect traceability or documentation

## 3. Responsibilities

| Role | Responsibility |
| --- | --- |
| Lab Manager | Authorize containment and initiate investigation |
| Lab Supervisor | Lead root cause analysis and maintain NCW register |
| Lab analyst/ operator | Report and document non-conforming work immediately |
| Operations Manager | Approve final corrective actions and ensure traceability of decisions |

4. Identification of non-conformity

Sources may include:

- Observation during routine work or supervision

- Internal audits

- Stability/homogeneity test failures

- Customer complaints

- Out-of-spec test results

- Incorrect CoA’s or labelling errors

5. Initial Actions

- Immediately halt use of affected RM/CRM or process step.

- Label the product or isolate the equipment with "NON-CONFORMING" tags.

- Notify the Laboratory/Operations Manager.

- Begin an NCW Report Form (paper).

## 6. Documentation

Each NCW must be documented using a controlled form/log that includes:

- Unique NCW ID

- Date/time of detection

- Process stage affected

- Description of non-conformance

- Responsible person(s)

- Immediate action taken (containment)

- Signature of person reporting

7. Evaluation and Risk Assessment

Assess whether the non-conformance:

- Affects previously released RMs/CRMs

- Requires rework, retesting, or full recall

- Risks invalidating assigned property values or traceability

- Has occurred before (recurring issue)

Use a risk matrix to evaluate severity and urgency.

## 8. Corrective Actions

Once root cause is confirmed:

- Define the correction and corrective action

- Assign responsible personnel and target dates

- Validate effectiveness after implementation

- Document updates to SOPs, training, or process controls if needed

## 9. Follow-Up and Closure

- Verify the corrective action eliminated or mitigated the issue

- Retest or recharacterise RM/CRM batch if necessary

- Document closure with review sign-off from operations/lab manager

- Update NCW register and archive all records securely

## 10. Record Retention

- NCW records must be stored for minimum of 10 years, linked to batch records.

- Include them in management review for trend analysis.