# ISO 18283 COMPLIANT SAMPLING PLAN

Hard Coal and Coke – Manual Sampling

Document Control

Procedure ID: AS008IW

Method Reference: ISO 18283:2006

Linked to ISO/IEC 17025:2017 Clauses 7.3 and 7.8.5

1. JOB & SAMPLE IDENTIFICATION

| Field | Entry |
| --- | --- |
| Job / Report Number | _______________________ |
| Date of Sampling | 20___-____-____ |
| Time of Sampling | Start Time: _______ End: _______ |
| Sampler Name(s) | _________________________________ |
| Customer Name | _______________________ |
| Location / Site | _______________________(Attach diagram if stockpiles vary) |
| Material Description | ☐ Washed Coal ☐ Unwashed Coal ☐ Coke |
| Nominal Top Size (mm) | [_______ mm] (Critical for Table 2 & 3 selection) |

2. SAMPLING SCHEME DESIGN

(Requirement: ISO 18283 Tables 1-3. The sampler must determine mass and increments based on the lot size.)

A. Lot Characteristics

- Total Lot Mass (tons): _________

- Sub-lots: Is the lot divided into sub-lots? ☐ Yes (Qty: ___) ☐ No

B. Number of Increments (n)

Refer to Table 1 (Images 20260129_114223, 114238, 114249).

- Target Precision (PL): ☐ 0.05 (High) ☐ 0.1 (General) ☐ 0.2 (Routine)

- Required Number of Increments: [_______]

- Guidance: For a 1,000t lot of unwashed coal (VI=10) targeting 0.1 precision, use 32-50 increments depending on sub-lots.

C. Mass of Primary Increment (m)

Refer to Table 2

- Minimum Mass per Increment: _______ kg

- Guidance: If Top Size is 50mm, min increment is 3 kg. If 100mm, min increment 6kg. If 150mm, min increment is 9 kg.

NB: Sampler opening ≥ 2.5 × top size (ISO 18283 Clause 5.3).

D. Total Composite Sample Mass (sample)

- Calculated Mass: (n increments) × (m kg/increment) = [_______ kg]

- Verification: Does this exceed the minimum mass in Table 3?

  - Check: For 50mm coal, General Analysis requires 170 kg (common) or 35 kg (moisture only).

- ☐ PASS (Calculated mass > Table 3 minimum)

- ☐ FAIL (Increase number of increments)

3. SAMPLING EXECUTION & METHODOLOGY

(Requirement: ISO 17025 Cl 7.3.2 - Selection and preparation method)

Method Used:

☐ Time-Basis Sampling (Increments taken every X minute)

☐ Mass-Basis Sampling (Increments taken every X ton)

☐ Stratified Random Sampling (Random selection within defined strata)

Equipment Used:

☐ Automatic Sampler (ID: ________)

☐ Manual Ladle / Shovel (Dimensions: ________ mm opening)

Note: Opening must be at least 2.5x the nominal top size of the coal.

4. ENVIRONMENTAL CONDITIONS

| Condition | Observation | Impact on Validity? |
| --- | --- | --- |
| Precipitation | ☐ None ☐ Rain ☐ Snow | High moisture risk |
| Wind | ☐ Calm ☐ Moderate ☐ High | Loss of fines (dust) |
| Contamination | ☐ Clean Area ☐ Dust Present | Ash bias risk |

5. SAMPLE HANDLING & PRESERVATION

Composite sample sealed and labelled.

Protected from contamination and moisture loss.

6. DEVIATIONS & EXCLUSIONS

(Requirement: ISO 17025 Cl 7.8.5 - Additions, deviations, or exclusions)

- Did the sampling follow the plan exactly? ☐ Yes ☐ No

- If NO, describe deviation: (Example: "Access to full stockpile restricted; sampled only accessible front face. Results relate only to accessible portion.")

7. CHAIN OF CUSTODY

| Action | Name | Signature | Date/Time |
| --- | --- | --- | --- |
| Sampled By: |  |  |  |
| Transported By: |  |  |  |
| Received at Lab: |  |  |  |

8. ATTACHMENTS

Stockpile diagram

Photographs

Weigh records

"How did you decide on the Number of Increments?"

- Answer: We used Table 1. For a generic lot of washed coal, we selected the column for PL=0.1 (Precision), which dictates the number of sub-lots and increments based on the total tonnage.

"How do you know your sample size is sufficient?"

- Answer: We reference Table 3 (Minimum Mass). For example, for 50mm Nominal Top Size, the standard requires a minimum of 170kg for general analysis. Our plan ensures n times m exceeds this value.

"How do you calculate precision?"

- Answer: We use the method in Clause 4.5, calculating PL using the preparation variance (V{PT}) and variance of increments (VI. We also perform replicate sampling periodically as described in Clause 4.6.2."

"Where is the sampling diagram?"

- Answer: "As per ISO 17025 Cl 7.8.5(c), we attach a sketch of the stockpile/conveyor belt to this plan, marking exactly where increments were taken to ensure representativeness."

TECHNICAL JUSTIFICATION (FOR AUDIT)

- Number of increments determined using ISO 18283 Table 1.

- Sample mass verified against ISO 18283 Table 3.

- Precision controlled using Clause 4.5 and replicate sampling.

- Representativeness demonstrated by attached diagram.