| AzurESky Coal Laboratory Technical Competency Evaluation System Standard: ISO/IEC 17025:2017 Cl. 6.2 | Document: AZCA-015 Rev 01 Module: Total Moisture (AS015IW Rev 01) Governing Standard: ISO 589 / ISO 13909-4 |
| --- | --- |
| TECHNICAL COMPETENCY ASSESSMENT: DETERMINATION OF TOTAL MOISTURE (SINGLE-STAGE & TWO-STAGE THERMAL DRYING) |  |

| Candidate Name: |  | Staff Number: |  |
| --- | --- | --- | --- |
| Assessment Date: |  | Attempt Number: | [ ] 1st Attempt [ ] 2nd Re-Assessment |
| Assessor Name: |  | Assessor Title: | Laboratory Supervisor / Tech Signatory |

MANDATORY ASSESSMENT RULES & QUALIFICATION CRITERIA

1. Purpose & Outcome: This assessment verifies technical competence in conducting Single-Stage and Two-Stage Total Moisture determinations in coal per procedure AS015IW Rev 01, ISO 589:2008, and ISO 13909-4:2025.
2. Total Marks & Pass Mark: The assessment consists of 25 points. To be declared COMPETENT, the candidate must achieve a minimum of 80% (20 / 25 points).
3. CRITICAL PROCESS GATES (Zero Tolerance): Questions *5 and *6 represent critical process control points. Candidates must achieve 100% on both Question *5 and Question *6. If either question is incorrect, the assessment is STOPPED IMMEDIATELY and the outcome is declared NOT YET COMPETENT.
4. Permitted Materials: Candidate may reference controlled test procedure AS015IW Rev 01 and raw data sheets AS015TF-01/02 during the assessment.

Question 1 [2 Marks]

Name the two thermal drying methods described in AS015IW for total moisture determination. Under what physical sample conditions is a laboratory required to perform a Two-Stage determination rather than a Single-Stage determination?

Question 2 [2 Marks]

In accordance with ISO 13909-4:2025 Table 1 and AS001IW Clause 4.1:
a) What is the minimum mass of the divided bulk sample required for a 50 mm nominal top-size coal sample?
b) What is the maximum nominal top-size accepted by the AzureSky coal laboratory?

Question 3 [2 Marks]

Describe the Cone and Quartering primary division procedure. If the mass of the chosen single quarter is less than the mandatory minimum mass, what specific action must the analyst take to ensure representativeness?

Question 4 [2 Marks]

At what temperature may the sample be air-dried during Stage 1? Explain the scientific rationale: why must the drying temperature strictly NOT exceed 40 °C, and what must be done if a heated oven is used before re-weighing?

Question *5 [5 Marks — CRITICAL GATE 1: ZERO TOLERANCE]

List the 5 key procedural steps in exact sequential order from Stage 1 air drying through to the preparation and weighing of duplicate test pans:
Step 1: _____________________________________________________________________________________
Step 2: _____________________________________________________________________________________
Step 3: _____________________________________________________________________________________
Step 4: _____________________________________________________________________________________
Step 5: _____________________________________________________________________________________

Question *6 [2 Marks — CRITICAL GATE 2: ZERO TOLERANCE]

State the exact operating temperature range for the moisture oven. In a Two-Stage determination (-3 mm), what is the initial drying duration, and what is the stability criterion between 30-minute intervals to declare the sample stable?

Question 7 [1 Mark]

What is the maximum allowable drying time in the oven before the test is marked as 'Failed' in LIMS? What mandatory action must be taken if a sample fails to stabilize within this time?

Question 8 [1 Mark]

How does the oven drying procedure for Single-Stage moisture (-10 mm) differ from Two-Stage moisture (-3 mm) regarding the initial 3-hour check? State the weighing requirement upon removing the hot tray from the oven.

Question 9 [2 Marks]

According to AS023TF and AS001IW Clause 4.1:
a) How is the moisture drying oven verified daily, and what is the operating tolerance limit?
b) On what schedule must crusher sieve efficiencies be checked for 25 mm/10 mm coarse crushers and 3 mm/4.75 mm fine crushers?

Question 10 [2 Marks]

Write the formula for calculating Surface (Free) Moisture percentage (Mar) with defined variables.
Calculate Mar for a bulk sample received at m1 = 35.00 kg and air-dried to m2 = 32.55 kg (show all working):

Question 11 [2 Marks]

During duplicate testing of residual moisture, Pan A yields Mres = 4.20% and Pan B yields Mres = 4.58%.
a) Does this duplicate pair comply with the ISO 589 repeatability limit? (State the limit and show calculation).
b) What automated alert is triggered in LIMS, and what mandatory action must the analyst take?

Question 12 [2 Marks]

Explain how AzureSky LIMS calculates Total Moisture compared to how ISO 589 calculates it for duplicate pans. Are both methods accepted under AzureSky QMS, and why?

ASSESSMENT OUTCOME, FEEDBACK & DECLARATION SIGN-OFF

| Total Candidate Score: | _______ / 25 Marks ( _____ % ) |
| --- | --- |
| Critical Gates Status: | Question *5 (Procedural Flow): [ ] PASS [ ] FAIL Question *6 (Temp & Timing): [ ] PASS [ ] FAIL |
| Competency Determination: | [ ] COMPETENT (Score ≥ 20/25 AND both Critical Gates Passed) [ ] NOT YET COMPETENT (Score < 20 OR any Critical Gate Failed) |
| Assessor Technical Comments & Recommendations: |  |
| Candidate Feedback & Acknowledgement: |  |
| Assessor Declaration: I hereby certify that the candidate has been assessed in accordance with AZCA-015 and ISO/IEC 17025:2017 Clause 6.2. | Assessor Name: __________________________ Signature: __________________________ Date: __________________________ |
| Candidate Declaration: I declare that I have received feedback on my assessment and understand the outcome. | Candidate Name: __________________________ Signature: __________________________ Date: __________________________ |