| AzurESky "secure quality of coal" | RAW DATA RECORD: AS015TF-01 SINGLE-STAGE TOTAL MOISTURE (NOMINAL TOP-SIZE -10 mm) | Form No: AS015TF-01 Test Method: AS015IW Cl. 4.2 Standard: ISO 589:2008 (Method B2) Effective Date: 02/2026 Revision Status: 00 |
| --- | --- | --- |

| 1. SAMPLE IDENTIFICATION & TRACEABILITY |  |  |  |
| --- | --- | --- | --- |
| Sample ID / Barcode: |  | LIMS Job / Batch No: |  |
| Client / Source / Pit: |  | Sample Type / Seam: |  |
| Date & Time Sampled: |  | Date & Time Received: |  |
| Nominal Top Size: | -10 mm | Crusher ID (-10 mm): |  |

| 2. EQUIPMENT IDENTIFICATION & PRE-TEST VERIFICATION |  |  |  |
| --- | --- | --- | --- |
| Analytical Balance ID: |  | Daily Balance Check (1000.0g): | [ ] Pass Value: ________ g |
| Drying Oven ID: |  | Oven Set Point / Range: | 105 °C – 110 °C (Target: 107 °C) |
| Thermocouple Calib. No: | (Ref. AS023TF) | Oven Daily Verification: | [ ] In Tolerance Temp: _____ °C |

| 3. SINGLE-STAGE OVEN DRYING MEASUREMENTS (AS015IW Clause 4.2) |  |  |  |
| --- | --- | --- | --- |
| Parameter / Measurement Field | Unit | DUPLICATE PAN A | DUPLICATE PAN B |
| Drying Pan Identification | — |  |  |
| Mass of Empty, Clean, Dry Pan Tare (m1) | g |  |  |
| Mass of Pan Tare + Sample Before Drying (m2) | g |  |  |
| Initial Sample Net Mass (m2 − m1) (Target: ~1000 g) | g |  |  |
| Time In Oven (105 °C to 110 °C) | hh:mm |  |  |
| Total Oven Drying Duration (Full 5 hours required) | hours | 5.0 hrs | 5.0 hrs |
| Time Out of Oven (Weigh hot within 5 minutes) | hh:mm |  |  |
| Mass of Pan Tare + Sample After Drying (m3) | g |  |  |
| Moisture Loss in Mass (m2 − m3) | g |  |  |
| Total Moisture Percentage (TM) = [(m2 − m3) / (m2 − m1)] × 100 | % |  |  |

| 4. QUALITY CONTROL & PRECISION COMPLIANCE (ISO 589 TABLE 1 / AS015IW CL. 6) |  |  |  |
| --- | --- | --- | --- |
| Duplicate Difference ¦TMA − TMB¦: | ____________ % | Repeatability Limit: | ≤ 0.50 % |
| Compliance Decision: | [ ] PASS [ ] REPEAT ALERT | Reported Mean Result (TM): (Reported to nearest 0.1 %) | ____________ % |
| Note: If duplicate difference > 0.50 %, sample fails repeatability criteria. An Out-of-Specification / Repeat Alert is triggered in LIMS, and the sample must be re-split and re-tested. |  |  |  |

| 5. AUTHORIZATION & APPROVAL |  |
| --- | --- |
| Analyst Performing Test: Name: ____________________________________ Signature: ________________________________ Date: ____________ | Supervisor / Senior Analyst Review: Name: ____________________________________ Signature: ________________________________ Date: ____________ |